# LED Illuminator

#### Library Specification

Version: 1.0

06 January 2003

Production Systems Engineering and Technology

## Eastman Kodak Company

1669 Lake Avenue

Rochester, New York 14652-4318

#### TABLE OF CONTENTS

Scope	3

Introduction	3

Command Set	3

Miscellaneous	4

Illuminator Information	4

Illuminator Settings (for use with LEDi application)	4

File Paths	4

Photodiode Values	4

Advanced Mode Functions (password required)	4

blSetIlluminatorCommunications	6

Camera Mode Parameters	7

blSetIlluminatorConfigurationRegistryKey	7

blSetIlluminatorMaxSceneHighlightPercent	7

blSetIlluminatorAvgSceneReflectancePercent	7

blSetIlluminatorExposureIndexOperation	8

blSetIlluminatorOperatingCodeValuePercent	8

blSetIlluminatorLensFNumber	8

blSetIlluminatorMaxCodeValue	8

blSetIlluminatorExposureTime	9

Mode	10

blSetIlluminatorModeRepetitive	10

blSetIlluminatorModeSingleShot	10

blSetIlluminatorModeGated	10

blSetIlluminatorModeContinuous	10

blIsIlluminatorModeRepetitive	11

blIsIlluminatorModeSingleShot	11

blIsIlluminatorModeGated	11

blIsIlluminatorModeContinuous	12

blIsIlluminatorModeSingleShotMemory	12

Camera Mode	13

blEnableIlluminatorCameraMode	14

blDisableIlluminatorCameraMode	14

Frequency	15

blSetIlluminatorRepetitiveFrequency	15

blSetIlluminatorGatedFrequency	15

dGetIlluminatorRepetitiveFrequency	15

dGetIlluminatorGatedFrequency	15

Pulse Width or Exposure Value	17

blSetIlluminatorSingleShotPulseWidth	17

blSetIlluminatorSingleShotExposureValue	17

dGetCurrentSingleShotPulseWidth	17

dGetCurrentSingleShotExposureValue	17

dGetCurrentRepetitiveExitPortLuminance	18

dGetMaximumSingleShotPulseWidth	18

dGetMinimumSingleShotPulseWidth	18

dGetMaximumSingleShotExposureValue	18

dGetMinimumSingleShotExposureValue	19

dGetMaximumGatedExposureValue	19

dGetMinimumGatedExposureValue	19

Cycles for Gated Mode	20

blSetIlluminatorGatedCycles	20

Illuminant	21

blSetIlluminatorIlluminantCIEA	21

blSetIlluminatorIlluminantCIEB	21

blSetIlluminatorIlluminantCIEC	21

blSetIlluminatorIlluminantCIED	21

blSetIlluminatorIlluminantCIET	22

blSetIlluminatorIlluminantPlanckian	22

blSetIlluminatorIlluminantCustom	22

blGetIlluminantName	23

iGetMaxNumbIlluminants	23

iGetMaxIlluminantName	23

Output Level	24

blSetIlluminatorLuminousOutputLevel	24

blSetIlluminatorRadiantOutputLevel	24

Operation	25

blEnableIlluminator	25

blDisableIlluminator	25

Test Chart Synthesis	26

blSetIlluminatorTestChart	26

blSetIlluminatorTestChartPatchName	26

blSetIlluminatorTestChartPatchIndex	26

blIsTestChartPatchIndexNeutral	26

blGetCurrentTestChartName	27

blGetCurrentTestChartPatchName	27

blGetTestChartName	27

blGetTestChartPatchName	28

iGetMaxNumbTestCharts	28

iGetMaxTestChartName	28

iGetMaxNumbTestChartPatches	28

iGetMaxTestChartPatchName	29

Lens Synthesis	30

blSetIlluminatorLens	30

blGetLensName	30

iGetMaxNumbLenses	30

iGetMaxLensName	30

Error Handling	32

blGetLastIlluminatorError	32

LED Calibration	33

blCalibrateLED	33

blPerformPhotoDiodeCalibration	33

Automatic Level Control	35

blSetIlluminatorALCAverage	35

blSetIlluminatorALCRepetiveCycles	35

blEnableIlluminatorALC	35

blDisableIlluminatorALC	35

Operational Scenarios	37

Scenario 1 – Repetitive Mode, Luminance	37

Scenario 2 – Single Shot, Exposure	38

Scenario 3 – Single Shot, Pulse Width	39

Scenario 4 – Single Shot, Exposure, Color Patch	40

Scenario 5 – Single Shot, Pre-Calculated Exposure Series	41

Scenario 6– Single Shot, Pre-Calculated Color Patch Series	43

##### Scope

This document will provide a description of each of the functions provided by the LED Illuminator Interface Library. The intended audience is the LED Illuminator development community as well as potential customers who plan to develop the software necessary to integrate the LED Illuminator into their test applications.

##### Introduction

The LED Illuminator communicates with a remote host via an I²C interface using the CommDLL.dll.

When loaded the library will attempt to establish communications with the LED Illuminator.  For this to occur successfully the following is required:

- The LED Illuminator must be powered on.

- The Calibre I²C PCI card has been installed in the PC and the software drivers have been loaded OR

- The National Instruments PIO-96 card has been installed in the PC and you are using a fixture interface board to talk to the illuminator.

- The communications cable between the PC and LED Illuminator must be connected.

- The CommDLL has been installed.

- By default, the data files must be located in the folder … C:\Program Files\Eastman Kodak Co\LED Illuminator

Each function will return a Boolean TRUE on success or a FALSE on failure.  The error reason may be found using blGetLastIlluminatorError() which will return a string describing the last error.

##### Command Set

Configuration

The LED Illuminator has many configuration options and several operating modes.  To support this flexibility there are many configuration parameters and corresponding methods to get the current parameter value and set a new value.  The configuration parameters are stored in the registry at the following key location: HKEY_CURRENT_USER\Software\Kodak\LED Illuminator.  This path may be changed with the command blSetIlluminatorConfigurationRegistryKey(char *szRegistryKey).  The configuration parameters are summarized in Table 1.

| Parameter | Method | Default | Description |
| --- | --- | --- | --- |
| Product Lens | blSetIlluminatorLens () | None | Sets the lens spectral response data set |
| Test Chart | blSetIlluminatorTestChart () | None | Sets the test chart spectral response data set |
| Test Patch Name | blSetIlluminatorTestChartPatchName | None |  |
| Illuminant Source | blSetIlluminatorIlluminantCIEA blSetIlluminatorIlluminantCIEB blSetIlluminatorIlluminantCIEC blSetIlluminatorIlluminantCIED blSetIlluminatorIlluminantCIET blSetIlluminatorIlluminantPlanckian | D-5500 |  |
| Max Scene Highlight Percent | blSetIlluminatorMaxSceneHighlightPercent | 105 |  |
| Average Scene Reflectance Percent | blSetIlluminatorAvgSceneReflectancePercent | 18 |  |
| Exposure Index Operating | blSetIlluminatorExposureIndexOperation | 115 |  |
| Max Code Value | blSetIlluminatorMaxCodeValue | 255 |  |
| Operating Code Value Percent | blSetIlluminatorOperatingCodeValuePercent | 80 |  |
| Lens F Number | blSetIlluminatorLensFNumber | 5.6 |  |
| Repetition Frequency | blSetIlluminatorRepetitiveFrequency | 2.0 |  |
| Correlated Color Temperature | blSetIlluminatorIlluminantCIED blSetIlluminatorIlluminantCIET blSetIlluminatorIlluminantPlanckian | 5500 |  |
| Exposure Time | blSetIlluminatorExposureTime | 20 |  |
| Exit Port Luminance | blSetIlluminatorLuminousOutputLevel | 400.0 |  |
| Single Shot Exposure Value | blSetIlluminatorSingleShotExposureValue | 175.0 |  |
| Single Shot Pulse Width | blSetIlluminatorPulseWidth | 0.41 |  |
| Gated Level Value | blSetIlluminatorGatedOutputLevel | 175 |  |
| Gated Frequency | blSetIlluminatorGatedFrequency | 40.0 |  |
| Gated Number Cycles | blSetIlluminatorGatedCycles | 400.0 |  |
| Output Level Control | blSetIlluminatorLuminousOutputLevel blSetIlluminatorRadiantOutputLevel | 1 |  |
| Output Units Selected |  | 0 |  |
| Mode Selected | blSetIlluminatorModeRepetitive() blSetIlluminatorModeSingleShot() blSetIlluminatorModeGated() | 0 |  |
| Exit Port Luminance Units |  | Cd/m2 (nit) |  |
| Single Shot Exp Units |  | Cd*sec/m2 |  |
| Pulse Width Units |  | Seconds |  |
| Gated Level Units |  | Cd*sec/m2 |  |

Table 1

###### Miscellaneous

###### Illuminator Information

###### Illuminator Settings (for use with LEDi application)

###### File Paths

###### Photodiode Values

###### Advanced Mode Functions (password required)

Communications

blSetIlluminatorCommunications

Format:	BOOL	blSetIlluminatorCommunications ( )

Parameters:	None

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description: Sets the mechanism or channel by which the PC will communicate to the Illuminator.  The routine examines entries in the Registry to establish communications with the Illuminator.

Format:	BOOL	blSetIlluminatorDataPath (char *szDataFilePath )

Parameters:	char *	szDataFilePath	Character string that specifies new data file path.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description: Sets the data path in the registry for the illuminator data files.

Default data path is "C:\\Program Files\\Eastman Kodak Co\\LED Illuminator\\".

Format:	BOOL	blSetIlluminatorErrorPath (char *szErrorFilePath )

Parameters:	char *	szErrorFilePath	Character string that specifies new error file path.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description: Sets the data path in the registry for the illuminator error files.

Default error path is "C:\\Program Files\\Eastman Kodak Co\\LED Illuminator\\ErrLog.txt".

###### Camera Mode Parameters

blSetIlluminatorConfigurationRegistryKey

Format:	BOOL	blSetIlluminatorConfigurationRegistryKey(char *szRegistryKey )

Parameters:	char *	szRegistryKey	Character string that specifies new key location.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description: Sets the registry key for the storage of all illuminator configuration values.  If the key does not exist, it will be created and default values will be generated.  Multiple keys may be created.  This is particularly useful if different applications or products using the illuminator require different configuration settings. .  For example:”HKEY_CURRENT_USER\Software\Kodak\ProductX\”.

Default key is "Software\\Kodak\\LED Illuminator\\".

This routine should be called before establishing communications with the blSetIlluminatorCommunications routine.

blSetIlluminatorMaxSceneHighlightPercent

Format:	BOOL	blSetIlluminatorMaxSceneHighlightPercent(__int32 iPercent )

Parameters:	__int32	iPercent	Desired percentage.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Sets the max scene highlight in percent (i.e. 100% would be 100).  Used only when illuminator is based on exposure.  Default value is 105 percent.

blSetIlluminatorAvgSceneReflectancePercent

Format:	BOOL	blSetIlluminatorAvgSceneReflectancePercent(__int32 iPercent )

Parameters:	__int32	iPercent	Desired percentage.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Sets the average scene reflectance in percent (i.e. 100% would be 100).  Used only when illuminator is based on exposure.  Default value is 18 percent.

blSetIlluminatorExposureIndexOperation

Format:	BOOL	blSetIlluminatorExposureIndexOperation(__int32 iExposureIndex)

Parameters:	__int32	iExposureIndex	Desired exposure index.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Sets the Operating exposure index.  Used only when illuminator output is based on exposure.  Default value is 115.

blSetIlluminatorOperatingCodeValuePercent

Format:	BOOL	blSetIlluminatorOperatingCodeValuePercent(__int32 iPercent )

Parameters:	__int32	iPercent	Desired operating code value.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Sets the operating code value in percent (i.e. 100% would be 100).  Used only when illuminator output is based on exposure.  Default value is 80 percent.

blSetIlluminatorLensFNumber

Format:	BOOL	blSetIlluminatorLensFNumber (double dFNumber)

Parameters:	double	dFNumber	Desired F number.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Sets the F-number of the lens.  Used only when illuminator output is based on exposure.  Default value is 5.6.

blSetIlluminatorMaxCodeValue

Format:	BOOL	blSetIlluminatorMaxCodeValue (__int32 iMaxCodeValue)

Parameters:	__int32	iExposureIndex	Desired maximum code value.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Sets the Maximum Code Value.  Used only when illuminator output is based on exposure.  Default value is 255.

blSetIlluminatorExposureTime

Format:	BOOL	blSetIlluminatorExposureTime (double dCameraExposureTime)

Parameters:	double	dCameraExposureTime	Desired exposure time.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Sets the camera exposure time in m-sec.  Used only when illuminator is based on exposure.  Default value is 20 m-sec.

###### Mode

blSetIlluminatorModeRepetitive

Format:	BOOL	blSetIlluminatorModeRepetitive ( )

Parameters:	None

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Sets the operating mode of the illuminator to repetitive.

blSetIlluminatorModeSingleShot

Format:	BOOL	blSetIlluminatorModeSingleShot ( )

Parameters:	None

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Sets the operating mode of the illuminator to single shot.

blSetIlluminatorModeGated

Format:	BOOL	blSetIlluminatorModeGated ( )

Parameters:	None

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Sets the operating mode of the illuminator to gated.

blSetIlluminatorModeContinuous

Format:	BOOL	blSetIlluminatorModeContinuous ( )

Parameters:	None

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Sets the operating mode of the illuminator to continuous.

blIsIlluminatorModeRepetitive

Format:	BOOL	blIsIlluminatorModeRepetitive ( )

Parameters:	None

Returns:	BOOL	status	TRUE – mode is repetitive

FALSE – mode is not repetitive

Description:	Indicates if the current mode is repetitive.  NOTE: This reflects the action of the last blSetIlluminatorModexxx() command.  The mode does not actually change until the blMakeItSo() command is executed.  For example, if the illuminator is currently operating in single shot mode and the mode is changed to repetitive, this function will return a TRUE, when if fact, the illuminator will remain in single shot mode until blMakeItSo() is executed.

blIsIlluminatorModeSingleShot

Format:	BOOL	blIsIlluminatorModeSingleShot ( )

Parameters:	None

Returns:	BOOL	status	TRUE – mode is single shot

FALSE – mode is not single shot

Description:	Indicates if the current mode is single shot.  NOTE: This reflects the action of the last blSetIlluminatorModexxx() command.  The mode does not actually change until the blMakeItSo() command is executed.  For example, if the illuminator is currently operating in gated mode and the mode is changed to single shot, this function will return a TRUE, when if fact, the illuminator will remain in gated mode until blMakeItSo() is executed.

blIsIlluminatorModeGated

Format:	BOOL	blIsIlluminatorModeGated ( )

Parameters:	None

Returns:	BOOL	status	TRUE – mode is gated

FALSE – mode is not gated

Description:	Indicates if the current mode is gated.  NOTE: This reflects the action of the last blSetIlluminatorModexxx() command.  The mode does not actually change until the blMakeItSo() command is executed.  For example, if the illuminator is currently operating in repetitive mode and the mode is changed to gated, this function will return a TRUE, when if fact, the illuminator will remain in repetitive mode until blMakeItSo() is executed.

blIsIlluminatorModeContinuous

Format:	BOOL	blIsIlluminatorModeContinuous ( )

Parameters:	None

Returns:	BOOL	status	TRUE – mode is continuous

FALSE – mode is not continuous

Description:	Indicates if the current mode is continuous.  NOTE: This reflects the action of the last blSetIlluminatorModexxx() command.  The mode does not actually change until the blMakeItSo() command is executed.  For example, if the illuminator is currently operating in repetitive mode and the mode is changed to continuous, this function will return a TRUE, when if fact, the illuminator will remain in repetitive mode until blMakeItSo() is executed.

blIsIlluminatorModeSingleShotMemory

Format:	BOOL	blIsIlluminatorModeSingleShotMemory ( )

Parameters:	None

Returns:	BOOL	status	TRUE – mode is single shot memory

FALSE – mode is not single shot memory

Description:	Indicates if the current mode is single shot memory.  NOTE: This reflects the action of the last blSetIlluminatorModexxx() command.  The mode does not actually change until the blMakeItSo() command is executed.  For example, if the illuminator is currently operating in repetitive mode and the mode is changed to single shot memory, this function will return a TRUE, when if fact, the illuminator will remain in repetitive mode until blMakeItSo() is executed.

###### Camera Mode

The output level of the LED illuminator can be set directly by specifying the luminance level (cd/m2) in repetitive or continuous modes or the exposure value (nit-seconds) in single-shot and gated modes. Another way to set the output level of the illuminator is via the ‘Camera Mode’ parameters. These parameters are listed here for the single-shot mode:

- EI	Exposure Index  (this is the official term which replaces ‘ISO’ and ‘ASA’)

- f#	f-number of the lens or illuminating cone when an aperture-based illumination geometry is used

- max	maximum scene highlight (as a % exposure) to be retained in the captured image

- ave	average scene reflectance assumption (typically a value of 18% is used)

- CVmax	maximum digital code value available (corresponds to max )

- CV	desired code value

There are a few equations which form the basis for the Camera Mode calculations and these will be reviewed next. The exposure index for a digital camera is defined by ISO 12232 as given by Eq. (1).

EI=10Ha		(1)

where Ha is the average image-plane exposure in lux-sec and this corresponds to an 18% reflectance object in the scene. In order to saturate the imaging device, we need an exposure that is greater by the factor max / ave. We can call this the saturation exposure (Hsat) and define it according to Eq. (2).

Hsat=10⋅ρmaxρave⋅EIsat	lux-sec	(2)

where EIsat is the saturation exposure index. The term ‘base ISO’ has also been used internally in the Kodak DSC community to denote the saturation exposure index. Note that the overall gain is typically adjusted such that the maximum available A/D code value (CVmax) corresponds to max.

As an example in interpreting the use of these formulas, let’s assume that a particular camera is operating at an exposure index of EI = 100 (ISO = 100) and that we desire to retain scene highlights up to max = 150% in the captured images. According to Eq. (1), the imager will receive an exposure Ha = 100 milli-lux-seconds. Remember that this corresponds to an 18% reflectance object in the scene. Let’s also assume that this particular camera is not capable of operating at an exposure index of less than EI = 100 before scene highlights begin to saturate the imager. Therefore, the saturation exposure index (or base ISO) of this camera is also 100 and the saturation exposure (Hsat) according to Eq. (2) is  833 milli-lux-seconds. Note that, in this particular example, the camera was operating at EI  = EIsat.

When either a lens or an aperture are used to illuminate the imager, the image-plane illuminance and the LED illuminator sphere luminance are related according to Eq. (3).

Ev=π⋅Ls4⋅f2	lux	(3)

where Ls is the sphere wall luminance in cd/m2 and f# is the f-number of the illuminating cone or the f-number of the lens.  We can combine Eqs. (2) and (3) and write them to solve for the required image-plane exposure as shown in Eq. (4).

H=40⋅ρmax⋅f2⋅CVπ⋅ρave⋅EIsat⋅CVmax	nit-sec.	(4)

Note that the illuminator controls exposure in single-shot mode by varying the overall on time. Therefore, it is important to ensure that the longest illuminator on-time fits within the available exposure time of the imager. Any of the independent parameters in Eq. (4) can now be used to control the required image-plane exposure without having to explicitly calculate and specify that exposure value.

blEnableIlluminatorCameraMode

Format:	BOOL	blEnableIlluminatorCameraMode ( )

Parameters:	None

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Enables the camera mode of operation. The camera mode uses the parameters Max Scene Highlight Percent, Average Scene Reflectance Percent, Exposure Index Operating, Max Code Value, Operating Code Value Percent and Lens F Number to calculate the luminance output as exposure time relative to the camera.   The camera mode may only be enabled in the single shot and repetitive modes.

blDisableIlluminatorCameraMode

Format:	BOOL	blDisableIlluminatorCameraMode ( )

Parameters:	None

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Disables the camera mode of operation.  By default the camera mode is disabled.

###### Frequency

These

blSetIlluminatorRepetitiveFrequency

Format:	BOOL	blSetIlluminatorRepetitiveFrequency (double dFrequency)

Parameters:	double	dFrequency	Desired operating frequency.

Returns:	BOOL	blStatus	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Sets the operating frequency for the LED illuminator in repetitive mode.  The default operating frequency is 10.0 kHz.

blSetIlluminatorGatedFrequency

Format:	BOOL	blSetIlluminatorGatedFrequency (double dFrequency)

Parameters:	double	dFrequency	Desired operating frequency.

Returns:	BOOL	blStatus	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Sets the operating frequency for the LED illuminator in gated mode.  The default operating frequency is 10.0 kHz.

dGetIlluminatorRepetitiveFrequency

Format:	double	dGetIlluminatorRepetitiveFrequency ()

Parameters	None

Returns:	double	dRepetitiveFrequency	Current repetitive operating frequency

Description:

dGetIlluminatorGatedFrequency

Format:	double	dGetIlluminatorGatedFrequency ()

Parameters	None

Returns:	double	dGatedFrequency	Current gated operating frequency

Description:

###### Pulse Width or Exposure Value

blSetIlluminatorSingleShotPulseWidth

Format:	BOOL	blSetIlluminatorSingleShotPulseWidth (double dPulseWidth)

Parameters:	double	dPulseWidth	Desired pulse width.

Returns:	BOOL	blStatus	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Sets the normal pulse width in μ-sec, for the LED illuminator in single shot mode.  The default pulse width is 40 μ-sec.

blSetIlluminatorSingleShotExposureValue

Format:	BOOL	blSetIlluminatorSingleShotExposureValue (double dExposureValue)

Parameters:	double	dExposureValue	Desired exposure value.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Sets the normal exposure value for the LED illuminator in single shot mode.  The default exposure value is 175.

Note: When in Single Shot Mode, the Illuminator either works with Pulse Width values or Exposure values, but not both at the same time.

dGetCurrentSingleShotPulseWidth

Format:	double	dGetCurrentSingleShotPulseWidth ()

Parameters:	None

Returns:	double	dPulseWidth	Pulse Width in μ -sec.

Description:	Gets the current pulse width in μ-sec, for the LED illuminator in single shot mode.

dGetCurrentSingleShotExposureValue

Format:	double	dGetCurrentSingleShotExposureValue ()

Parameters:	None

Returns:	double	dExposureValue	Exposure Value in cd-sec/m2 or nit-sec.

Description:	Gets the current exposure value in cd-sec/m2 or nit-sec, for the LED illuminator in single shot mode.

dGetCurrentRepetitiveExitPortLuminance

Format:	double	dGetCurrentRepetitiveExitPortLuminance ()

Parameters:	None

Returns:	double	dExitPortLuminance	Exit Port Luminance in cd/m2.

Description:	Gets the current exit port luminance for the illuminator in repetitive mode.

dGetMaximumSingleShotPulseWidth

Format:	double	dGetMaximumSingleShotPulseWidth ()

Parameters:	None

Returns:	double	dMaxPulseWidth	Pulse Width in μ -sec.

Description:	Gets the maximum pulse width in μ-sec, for the LED illuminator in single shot mode.

dGetMinimumSingleShotPulseWidth

Format:	double	dGetMinimumSingleShotPulseWidth ()

Parameters:	None

Returns:	double	dMinPulseWidth	Pulse Width in μ -sec.

Description:	Gets the minimum pulse width in μ-sec, for the LED illuminator in single shot mode.

dGetMaximumSingleShotExposureValue

Format:	double	dGetMaximumSingleShotExposureValue ()

Parameters:	None

Returns:	double	dExposureValue	Exposure Value in cd-sec/m2 or nit-sec.

Description:	Gets the maximum exposure value in cd-sec/m2 or nit-sec, for the LED illuminator in single shot mode.

dGetMinimumSingleShotExposureValue

Format:	double	dGetMinimumSingleShotExposureValue ()

Parameters:	None

Returns:	double	dExposureValue	Exposure Value in cd-sec/m2 or nit-sec.

Description:	Gets the minimum exposure value in cd-sec/m2 or nit-sec, for the LED illuminator in single shot mode.

dGetMaximumGatedExposureValue

Format:	double	dGetMaximumGatedExposureValue ()

Parameters:	None

Returns:	double	dExposureValue	Exposure Value in cd-sec/m2 or nit-sec.

Description:	Gets the maximum exposure value cd-sec/m2 or nit-sec, for the LED illuminator in gated mode.

dGetMinimumGatedExposureValue

Format:	double	dGetMinimumGatedExposureValue ()

Parameters:	None

Returns:	double	dExposureValue	Exposure Value in cd-sec/m2 or nit-sec.

Description:	Gets the minimum exposure value in cd-sec/m2 or nit-sec, for the LED illuminator in gated mode.

###### Cycles for Gated Mode

blSetIlluminatorGatedCycles

Format:	BOOL	blSetIlluminatorGatedCycles (__int32 iNumbCycles)

Parameters:	__int32	iNumbCycles	Desired number of cycles to output.

Returns:	BOOL	blStatus	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Sets the number of cycles to output in gated mode following a trigger.  Valid values are in the range from 1 to 16000 cycles. This parameter is ignored in repetitive and single-shot modes.  The default number of cycles is 400.

###### Illuminant

These commands set the spectral characteristics for the desired illuminant.

blSetIlluminatorIlluminantCIEA

Format:	BOOL	blSetIlluminatorIlluminantCIEA ( )

Parameters:	None

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description: Sets the illuminator spectral output to be type CIE A.

This routine is currently disabled.

blSetIlluminatorIlluminantCIEB

Format:	BOOL	blSetIlluminatorIlluminantCIEB ( )

Parameters:	None

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Sets the illuminator spectral output to be type CIE B.

blSetIlluminatorIlluminantCIEC

Format:	BOOL	blSetIlluminatorIlluminantCIEC ( )

Parameters:	None

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Sets the illuminator spectral output to be type CIE C.

blSetIlluminatorIlluminantCIED

Format:	BOOL	blSetIlluminatorIlluminantCIED (double dColorTemp)

Parameters:	double	dColorTemp	Desired color temperature.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Sets the illuminator spectral output to be type CIE D with the specified color temperature.  Valid range is 4,000 K to 10,000 K

blSetIlluminatorIlluminantCIET

Format:	BOOL	blSetIlluminatorIlluminantCIET (double dColorTemp)

Parameters:	double	dColorTemp	Desired color temperature.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Sets the illuminator spectral output to be type CIE T with the specified color temperature.  Valid range is 3,200 K to 10,000 K

blSetIlluminatorIlluminantPlanckian

Format:	BOOL	blSetIlluminatorIlluminantPlanckian (double dColorTemp )

Parameters:	double	dColorTemp	Desired color temperature.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Sets the illuminator spectral output to be type Planckian with the specified color temperature.  Valid range is 2,800 K to 10,000 K

This routine is currently disabled.

blSetIlluminatorIlluminantCustom

Format:	BOOL	blSetIlluminatorIlluminantCustom (char *szCustomName)

Parameters:	char	*szCustomName	A pointer to character string name of the desired illuminant.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Sets the illuminator spectral output to the curve as specified by the name of the illuminant.

This routine is currently disabled.

blGetIlluminantName

Format:	BOOL	blGetIlluminantName(__int32 iIndex, char *szName,  __int32 iStringLen)

Parameters:	__int32	iIndex			Index of the desired illuminant name.  The first index is 0.

char *szName			Pointer to memory in which the name will be placed.

__int32	iStringLen	Max number of characters that szName can accommodate.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Gets the illuminant name at the specified index.  All available illuminant names are stored in an array.  iGetMaxNumbIlluminants() returns the number of names in the array and iGetMaxIlluminantName() will return the length of the largest illuminant name.

iGetMaxNumbIlluminants

Format:	__int32	iGetMaxNumbIlluminants ()

Parameters:	None

Returns:	__int32	iMaxNumbIlluminants	Max number of illuminants available.

Description:	Gets the maximum number of illuminants currently installed and available to the user.

iGetMaxIlluminantName

Format:	__int32	iGetMaxIlluminantName ()

Parameters:	None

Returns:	__int32	iMaxIlluminantNameSize	Max number of characters.

Description:	Gets the maximum number of characters in any currently installed illuminant name.

###### Output Level

blSetIlluminatorLuminousOutputLevel

Format:	BOOL	blSetIlluminatorLuminousOutputLevel (double dLevel )

Parameters:	double	dLevel	Output level in cd/m2.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Sets the output level of the illuminator in repetitive mode to the luminous level specified.

blSetIlluminatorRadiantOutputLevel

Format:	BOOL	blSetIlluminatorRadiantOutputLevel (double dLevel)

Parameters:	double	dLevel	Output level in watts/sr-m2.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Sets the output level of the illuminator in repetitive mode to the radiant level specified.

###### Operation

blEnableIlluminator

Format:	BOOL	blEnableIlluminator ()

Parameters	None

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Enables or turns on the illuminator under the conditions specified by the last blMakeItSo().  If blMakeItSo() has not been previously called, this function will fail.

NOTE: In all modes, blMakeItSo() will execute blEnableIlluminator().  If you need to execute blMakeItSo() to allow for the checking of minimum or maximum settings, it will be necessary to perform a blDisableIlluminator() following the blMakeItSo().

blDisableIlluminator

Format:	BOOL	blDisableIlluminator ()

Parameters:	None

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Disables or turns off the illuminator.

Format:	BOOL	blMakeItSo()

Parameters	None

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description:	Applies the previously set parameters to the Illuminator.  All Illuminator calculations are done at this time.  Any problems found will be reported through the blGetLastIlluminatorError routine.

###### Test Chart Synthesis

blSetIlluminatorTestChart

Format:	BOOL	blSetIlluminatorTestChart (char *szTestChartName)

Parameters:	char *	szTestChartName	Desired test chart name.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description: Sets the test chart that is to be illuminated.  If no test chart is desired the TestChartName is “None”.

blSetIlluminatorTestChartPatchName

Format:	BOOL	blSetIlluminatorTestChartPatchName (char *szTestChartPatchName)

Parameters:	char *	szColorChartName	Desired patch name.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description: Selects a test patch by name from the selected test chart.

blSetIlluminatorTestChartPatchIndex

Format:	BOOL	blSetIlluminatorTestChartPatchIndex(__int32 iIndex )

Parameters:	__int32	iIndex	Desired patch index

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description: Selects a test patch by index from the selected test chart.  The first patch on a chart is 0.

blIsTestChartPatchIndexNeutral

Format:	BOOL	blIsTestChartPatchIndexNeutral(__int32 iIndex)

Parameters:	__int32	iIndex	Desired patch index

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description: Indicates if the specified test patch index is a neutral patch.  The first patch on a chart is 0.

blGetCurrentTestChartName

Format:	BOOL	blGetCurrentTestChartName(char *szTestChartName, __int32 iStringLen)

Parameters:	char *				Pointer to memory in which the name will be placed.

__int32	iStringLen	Max number of characters that szTestChartName can accommodate.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description: Gets the current test chart name.

blGetCurrentTestChartPatchName

Format:	BOOL	blGetCurrentTestChartPatchName(char *szTestChartPatchName, __int32 iStringLen)

Parameters:	char *				Pointer to memory in which the name will be placed.

__int32	iStringLen	Max number of characters that szTestChartPatchName can accommodate.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description Gets the current test patch name.

blGetTestChartName

Format:	BOOL	blGetTestChartName(__int32 iIndex, char *szTestChartName, __int32 iStringLen)

Parameters:	__int32	iIndex	Index of the desired test chart name.  The first index is 0.

char *				Pointer to memory in which the name will be placed.

__int32	iStringLen	Max number of characters that szTestChartName can accommodate.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description: Gets the test chart name at the specified index.  All available test chart names are stored in an array.  iGetMaxNumbTestCharts () returns the number of names in the array and iGetMaxTestChartName () will return the length of the largest test chart name.

blGetTestChartPatchName

Format:	BOOL	blGetTestChartPatchName(__int32 iIndex, char *szTestChartPatchName, __int32 iStringLen)

Parameters:	__int32	iIndex	Index of the desired test patch name.  The first index is 0.

char *				Pointer to memory in which the name will be placed.

__int32	iStringLen	Max number of characters that szTestChartPatchName can accommodate.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description: Gets the test patch name at the specified index.  All available test patch names for the current test chart are stored in an array.  iGetMaxNumbTestChartPatches () returns the number of names in the array and iGetMaxTestChartPatchName () will return the length of the largest test patch name.

iGetMaxNumbTestCharts

Format:	__int32	iGetMaxNumbTestCharts()

Parameters:	None

Returns:	__int32	iMaxNumbTestCharts	Max number of test charts available.

Description:	Gets the maximum number of test charts currently installed and available to the user.

iGetMaxTestChartName

Format:	__int32	iGetMaxTestChartName()

Parameters:	None

Returns:	__int32	iMaxTestChartNameSize	Max number of characters.

Description:	Gets the maximum number of characters in any currently installed test chart name.

iGetMaxNumbTestChartPatches

Format:	__int32	iGetMaxNumbTestChartPatches()

Parameters:	None

Returns:	__int32	iMaxNumbTestChartPatches	Max number of test patches available.

Description:	Gets the maximum number of test patches available on the current test chart.

iGetMaxTestChartPatchName

Format:	__int32	iGetMaxTestChartPatchName()

Parameters:	None

Returns:	__int32	iMaxTestChartPatchNameSize	Max number of characters.

Description:	Gets the maximum number of characters in any test patch name of the current test chart.

###### Lens Synthesis

blSetIlluminatorLens

Format:	BOOL 	blSetIlluminatorLens ( char *szLensDataName )

Parameters:	char	*szLensDataName	Desired lens name

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description: Selects a lens whose spectral response is to be convolved with the current illuminant. .  Set to

“None” for no Lens Data file being used.

blGetLensName

Format:	BOOL 	blGetLensName(__int32 iIndex, char *szLensName, __int32 iStringLen)

Parameters:	__int32	iIndex	Index of the desired lens.  The first index is 0.

char *				Pointer to memory in which the name will be placed.

__int32	iStringLen	Max number of characters that szLensName can accommodate.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Description: Gets the lens name at the specified index.  All available lens names are stored in an array.  iGetMaxNumbLenses () returns the number of names in the array and iGetMaxLensName () will return the length of the largest lens name.

iGetMaxNumbLenses

Format:	__int32	iGetMaxNumbLenses()

Parameters:	None

Returns:	__int32	iMaxNumbLenses	Max number of lenses available.

Description:	Gets the maximum number of lenses currently installed and available to the user.

iGetMaxLensName

Format:	__int32	iGetMaxLensName()

Parameters:	None

Returns:	__int32	iMaxLensNameSize	Max number of characters.

Description:	Gets the maximum number of characters in any lens name.

###### Error Handling

blGetLastIlluminatorError

Format:	char[] 	blGetLastIlluminatorError(char *szLastError, __int32 iStringLen)

Parameters	char *szLastError			Pointer to memory in which the message will be placed.

__int32	iStringLen	Max number of characters that szLastError can accommodate.

Returns:	BOOL	status	TRUE - success

FALSE – failure

Description: Gets the last error message in response to a failure of a previously executed command.

###### LED Calibration

Before the Illuminator is used for any meaningful testing, the software must have a current set of Photo Diode readings to work with.  This routine will force the illuminator to take a new set of photo diode readings.  The routine will come up with a message box that says:

The software will take a new set of photo diode readings.

Before taking the readings, the exit port opening of the

LED Illuminator should be covered so that the ambient room

lighting does not affect the readings.  Please cover the

exit port opening and then press the OK button to continue.

At this point the user should cover the exit port opening and then press the OK button.  The software will take the photo diode readings and put up another message box when the readings are complete.

blCalibrateLED

Format:	BOOL blCalibrateLED (__int32 iLEDAddress, BYTE bLEDCurrent, double dPulseWidth,

__int32 iNumbPhotodiodeVals, __int32 * piPhotodiodeVals )

Parameters:	__int32	iLEDAddress	Contains the address of the LED which is to be on.

All other LEDs will be turned off and their currents

set to zero. Bits 0-5 are the LED address within the

cluster and bits 6-7 are the cluster address.

BYTE bLEDCurrent	8-bit current setting for the addressed LED. 0 = 40.0

mA, 255 = 0 mA.

double	dPulseWidth	Determines the on-time of the LED following receipt

of an external trigger pulse. Valid values are in the

range from TBD usec. to TBD usec in TBD usec

increments.

__int32	NumbPhotodiodeVals	Specifies the number of photodiode readings to take

during the LED on-time. Readings will be evenly

spaced within the specified pulse width but cannot be

any closer than 2.5 usec apart.

__int32	*piPhotodiodeVals	Pointer to an array to receive the individual photo-

diode readings. The array must be large enough to

receive the requested number of samples.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

blPerformPhotoDiodeCalibration

Format:	BOOL 	blPerformPhotoDiodeCalibration ( )

Parameters:	None

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Sets the illuminator into a diagnostic mode, allowing each of the individual LEDs to be exercised.  The selected LED is turned on for the specified time immediately following receipt of an external trigger pulse. This allows the illuminator output to be sampled by a spectro-radiometer. This command overrides the normal operating modes of the illuminator.  This routine is currently disabled.

###### Automatic Level Control

This command allows the user to turn the ALC feature on or off and specify the number of consecutive photodiode readings to be averaged.

blSetIlluminatorALCAverage

Format:	BOOL	blSetIlluminatorALCAverage (__int32 iNumbValsToAverage)

Parameters:	 __int32	iNumbValsToAverage	Determines the number of photodiode readings to

average.   This routine is currently disabled.

This command allows the user to turn the ALC feature on or off and specify the number of consecutive photodiode readings to be averaged.

blSetIlluminatorALCRepetiveCycles

Format:	BOOL	blSetIlluminatorALCRepetiveCycles (__int32 iNumbCycles)

Parameters:	 __int32	iNumbCycles	Determines the number of cycles to wait in repetitive

mode before taking another set of photodiode

readings. This parameter is ignored in single-shot and

gated modes.  This routine is currently disabled.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

This command allows the user to enable the ALC feature.

blEnableIlluminatorALC

Format:	BOOL	blEnableIlluminatorALC ()	Enables the ALC feature, which means that the Illuminator

will use the current photo diode readings.

Parameters	None

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

This command allows the user to disable the ALC feature.

blDisableIlluminatorALC

Format:	BOOL	blDisableIlluminatorALC ()	Disables the ALC feature, which means that the Illuminator

will never take photo diode readings, it will always use the

set of calibration readings.

Parameters	None

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Photodiode Monitor

This command allows the current average photodiode readings to be monitored for the 18 LED channels.

Format:	int	get_photodiode_values ( short* photodiode_values )

Parameters:	short	*photodiode_values	Pointer to an array into which the photodiode

readings will be written.

Returns:	int	error	0 = no error

This routine is not currently implemented.

Calibration Data

This command allows the spectral data and photodiode readings for the individual LEDs to be written into a non-volatile memory area in the LED illuminator.

Format:	int	set_calibration_data ( double* spd_data, double* photodiode_data )

Parameters:	double	*spd_data	Pointer to an array containing the spectral power

distribution data for the individual LEDs.

double	*photodiode_data	Pointer to an array containing the photodiode

readings for the individual LEDs.

Returns:	int	error	0 = no error

This routine is not currently implemented.

Other Functions

Format:	BOOL	blGetIlluminatorSerialNumber (char *szSerialNumber, __int32 iStringLen)

Parameters	char *szSerialNumber			Pointer to memory in which the Serial Number will be

placed.

__int32 iStringLen	Maximum number of characters that szSerialNumber can accommodate.

Returns:	BOOL	status	TRUE - success

FALSE – failure (call blGetLastIlluminatorError() )

Functions yet to be implemented

- Get minimum output level

- Get maximum output level

- Get information (exit port diameter, calibration date, etc. )

- Set information

- Read current status (mode, illuminant, output level, etc., etc.)

- Save setup to memory ‘x’

- Read setup for memory ‘x’

##### Operational Scenarios

###### Scenario 1 – Repetitive Mode, Luminance

We would like to configure the illuminator to produce an illuminant of “Daylight 5500” at a luminous output level of 175 cd/m2.  We will be using a CMOS imager to capture an image with a 20-ms. integration time using a “rolling shutter” technique that results in the imager being progressively exposed.

Solution

Due to the need to expose the imager for 20-ms. the illuminator will be used in repetitive mode.  In this mode the illuminator will repeatedly produce the illuminant desired at a specified interval.  We will set the repetition frequency to 2 kHz, which will result in 40 cycles of the illuminator in the specified 20-ms integration time.

blSetIlluminatorConfigurationRegistryKey(“Software\Kodak\MyProduct”);	// Set the Registry Path

blSetIlluminatorCommunications();					// Enable communications

blSetIlluminatorLens(“None”);						// Do not simulate a lens in the optical path

blSetIlluminatorTestChart(“None”)					// No test chart

const	double	dColorTemp 	= 5500.0;

const 	double	dLevel		= 175.0;

const	double	dFrequency	= 2.0;

blSetIlluminatorModeRepetitive();						// Put Illuminator in Repetitive Mode

blSetIlluminatorIlluminantCIED(dColorTemp);				// Set up for Daylight 5500

blSetIlluminatorLuminousOutputLevel(dLevel);				// Set the Output Level to 175

blSetIlluminatorRepetitiveFrequency(dFrequency);				// Set the frequency to 2 kHz

blMakeItSo();								// Configure the Illuminator

Note: Setting the Configuration Registry Key and Communications only needs to be done once.  It is shown here for completeness.  Setting the Lens and the Test Chart to “None” is not necessary as these are the default setting, however if you do not know the last configuration of the illuminator, these two lines could save you much debug time.

###### Scenario 2 – Single Shot, Exposure

We would like to configure the illuminator to produce an illuminant of “Daylight 5500” with an exposure of 30.5 cd-sec/m2.  We will be using a CMOS imager to capture an image with a 20-ms. integration time using a “global shutter” technique that results in all pixels of the imager being integrate simultaneously.

Solution

Due to the simultaneous exposure of all pixels on the imager, the illuminator must be used in single shot mode.  In this mode the illuminator will produce the desire exposure over some finite period of time.  It will be necessary to check the maximum pulse width of the illuminator to insure that the exposure is complete within the specified 20-ms integration time.  The duration of the exposure may not be determined until after the MakeItSo command has been issued.  It is necessary to provide an external trigger to initiate the exposure.

blSetIlluminatorConfigurationRegistryKey(“Software\Kodak\MyProduct”);	// Set the Registry Path

blSetIlluminatorCommunications();					// Enable communications

blSetIlluminatorLens(“None”);						// Do not simulate a lens in the optical path

blSetIlluminatorTestChart(“None”)					// No test chart

const	double	dColorTemp 	= 5500.0;

const 	double	dLevel		= 30.5;

const  double	dIntegrationTime	= 0.020;					// Integration time in seconds

blSetIlluminatorModeSingleShot();						// Put Illuminator in Single Shot Mode

blSetIlluminatorIlluminantCIED(dColorTemp);				// Set up for Daylight 5500

blSetIlluminatorSingleShotExposureValue (dLevel);			// Set the Exposure to 30.5 cd-sec/m2

blMakeItSo();								// Configure the Illuminator

if(dGetMaximumSingleShotPulseWidth() > dIntegrationTime)		// Calculations are not performed until

{									//   after MakeItSo().

// Must select a lower exposure

}

Note: Setting the Configuration Registry Key and Communications only needs to be done once.  It is shown here for completeness.  Setting the Lens and the Test Chart to “None” is not necessary as these are the default setting, however if you do not know the last configuration of the illuminator, these two lines could save you much debug time.

###### Scenario 3 – Single Shot, Pulse Width

We would like to configure the illuminator to produce an illuminant of “Tungsten 4800” with an exposure lasting 9-ms. We do not care what the luminance level is as long as it can be determined.

Solution

To produce a single exposure of a predetermined duration the illuminator must be used in single shot mode.  In this mode the illuminator will produce an exposure over the specified period of time.  The exposure level may not be determined until after the MakeItSo command has been issued.  It is necessary to provide an external trigger to initiate the exposure.

blSetIlluminatorConfigurationRegistryKey(“Software\Kodak\MyProduct”);	// Set the Registry Path

blSetIlluminatorCommunications();					// Enable communications

blSetIlluminatorLens(“None”);						// Do not simulate a lens in the optical path

blSetIlluminatorTestChart(“None”)					// No test chart

const	double	dColorTemp 	= 4800.0;

const 	double	dPulseWidth	= 0.900;					// 900 milliseconds

const float	fIntegrationTime	= 0.020;					// Integration time in seconds

blSetIlluminatorModeSingleShot();						// Put Illuminator in Single Shot Mode

blSetIlluminatorIlluminantCIET(dColorTemp);				// Set up for Tungsten 4800

blSetIlluminatorPulseWidth (dPulseWidth);				// Set the Pulse Width to 9 m-sec

blMakeItSo();								// Configure the Illuminator

double dExposure = dGetCurrentSingleShotExposureValue();

Note: Setting the Configuration Registry Key and Communications only needs to be done once.  It is shown here for completeness.  Setting the Lens and the Test Chart to “None” is not necessary as these are the default setting, however if you do not know the last configuration of the illuminator, these two lines could save you much debug time.

When exiting your application, first call blDisableIlluminator to turn off the illuminator and then call blShutDownIlluminator to disconnect from the illuminator DLL.  This call will release any memory allocated by the DLL.

###### Scenario 4 – Single Shot, Exposure, Color Patch

We would like to configure the illuminator to produce an illuminant of “Daylight 5500” with an exposure of 30.5 cd-sec/m2 illuminating the twenty second patch on the “Kodak 64 Patch” chart. We will be using a CMOS imager to capture an image with a 20-ms. integration time using a “global shutter” technique that results in all pixels of the imager being integrate simultaneously.

Solution

Due to the simultaneous exposure of all pixels on the imager, the illuminator must be used in single shot mode.  In this mode the illuminator will produce the desire exposure over some finite period of time.  It will be necessary to check the maximum pulse width of the illuminator to insure that the exposure is complete within the specified 20-ms integration time.  The duration of the exposure may not be determined until after the MakeItSo command has been issued.  It is necessary to provide an external trigger to initiate the exposure.

blSetIlluminatorConfigurationRegistryKey(“Software\Kodak\MyProduct”);	// Set the Registry Path

blSetIlluminatorCommunications();					// Enable communications

blSetIlluminatorLens(“None”);						// Do not simulate a lens in the optical path

const	__int32	(iTWENTY_SECOND_PATCH	= 21;			// Patch index starts at zero

blSetIlluminatorTestChart(““Kodak 64 Patch”)				// Test chart

blSetIlluminatorTestChartPatchIndex(iTWENTY_SECOND_PATCH);	// Patch index

const	double	dColorTemp 	= 5500.0;

const 	double	dLevel		= 30.5;

const float	fIntegrationTime	= 0.020;					// Integration time in seconds

blSetIlluminatorModeSingleShot();						// Put Illuminator in Single Shot Mode

blSetIlluminatorIlluminantCIED(dColorTemp);				// Set up for Daylight 5500

blSetIlluminatorSingleShotExposureValue (dLevel);			// Set the Exposure to 30.5 cd-sec/m2

blMakeItSo();								// Configure the Illuminator

if(fGetIlluminatorMaximumPulseWidth() > fIntegrationTime)		// Calculations are not performed until

{									//   after MakeItSo().

// Must select a lower exposure

}

Note: Setting the Configuration Registry Key and Communications only needs to be done once.  It is shown here for completeness.  Setting the Lens to “None” is not necessary as this is the default setting, however if you do not know the last configuration of the illuminator, this line could save you much debug time.

Units should be consistent (i.e. all times in seconds or m-sec or u-sec, not mixed)

What with all the doubles? ColorTemp, should be an int, pulse width, exposures should be single

###### Scenario 5 – Single Shot, Pre-Calculated Exposure Series

We would like to configure the illuminator to produce an illuminant of “Daylight 5500” and to be able to rapidly change exposure from 0 cd-sec/m2 to a maximum of 30.5 cd-sec/m2  in 100 equal steps (0% to 100% exposure).  Our optical system is f4.0, scene highlights up to max = 150% is desired and the average scene reflectance ave = 18%.

Solution

Due to the desire to rapidly change exposure (possibly at the imager frame rate) the illuminator must be used in single shot mode, camera mode.  In this mode the illuminator will produce the desire exposure over some finite period of time.  The maximum pulse width will occur at the maximum exposure or the 100% exposure.  It will be necessary to check the maximum pulse width of the illuminator to insure that the integration time of the imager is sufficiently long at the 100% setting.  The duration of the exposure may not be determined until after the MakeItSo command has been issued.  It is necessary to provide an external trigger to initiate the exposure.

Note: We will have 100 steps from 1 to 100%.  When we want 0% we will disable the illuminator.

blSetIlluminatorConfigurationRegistryKey(“Software\Kodak\MyProduct”);	// Set the Registry Path

blSetIlluminatorCommunications();					// Enable communications

blSetIlluminatorLens(“None”);						// Do not simulate a lens in the optical path

blSetIlluminatorTestChart(“None”)					// No test chart

const double	dPie		= 3.141;

const	double	dColorTemp 	= 5500.0;

const 	double	dLevel		= 30.5;

const double 	dpmax		= 150.0;					// max scene highlights

const double	dpave		= 18.0;					// average scene highlights

const double	dFstop		= 4.0;					// f# of optical system

double	dExposureIndex;

double adIllumPulseWidth[iMAX_EXPOSURES];

double adllumExposure[iMAX_EXPOSURES];

// Solve equation 4 in the Camera Mode description for Exposure Index (EI).  We will calculate EI at the max code value

// hence CV = Cvmax and they cancel each other.

dExposureIndex = (40.0 * dpmax * dFstop * dFstop) / ( dLevel * dPie * dpave);

blSetIlluminatorModeSingleShot();						// Put Illuminator in Single Shot Mode

blSetIlluminatorIlluminantCIED(dColorTemp);				// Set up for Daylight 5500

// Enable camera mode and set camera mode parameters

blEnableIlluminatorCameraMode ();

blSetIlluminatorMaxSceneHighlightPercent(dpmax);

blSetIlluminatorAvgSceneReflectancePercent(dpave);

blSetIlluminatorExposureIndexOperation(dExposureIndex);

blSetIlluminatorLensFNumber(dFstop);

blSetIlluminatorMaxCodeValue(1023.0);

char	szError[iMAX_ERROR_LEN];

const	__int32	i100_PERCENT	= 100;

// Calculate and store Illuminator Pulse Width Array for exposure range of 0 to 30.5 cd-sec/m2.

for(iIndex = 1; iIndex <= i100_PERCENT; iIndex++)

{

blSetIlluminatorOperatingCodeValuePercent((double)iIndex);

if(blIlluminatorCalcPulseWidthArrayToMemory(iIndex) == FALSE)

{

blGetLastIlluminatorError(szError, iMAX_ERROR_LEN);

}

// If you would like to know the max pulse width and/or exposure value for each index

// then get them at this time.

{

// I will store the pulse widths in an array for future reference

adIllumPulseWidth[iIndex] = dGetCurrentSingleShotPulseWidth();

// I will also store the exposure in an array for future reference

adllumExposure[iIndex]  = dGetCurrentSingleShotExposureValue();

}

// If you only need to know the longest (and in this case the last) exposure duration

if(iIndex == i100_PERCENT)

{

m_dIllum100PerCentPulseWidth = dGetCurrentSingleShotPulseWidth();

// Convert pulse width from usec to msec

m_dIllum100PerCentPulseWidth = m_dIllum100PerCentPulseWidth * 1000.0;

// If we want to know the exposure for this configuration we can also read back the exposure

m_dIllum100PerCentExposure  = dGetCurrentSingleShotExposureValue();

}

}

// We wish to generate an exposure curve for an imager with 20 steps

for(iIndex = 5; iIndex <= i100_PERCENT; iIndex = iIndex + (i100_PERCENT / 20))

{

blIlluminatorDownloadPulseWidthsFromMemory(iIndex);

// Capture and image and analyze

}

###### Scenario 6– Single Shot, Pre-Calculated Color Patch Series

We would like to configure the illuminator to produce an illuminant of “Daylight 5500” and to be able to rapidly step through a 64 patch color chart.  Our optical system is f4.0, scene highlights up to max = 150% is desired and the average scene reflectance ave = 18%.

Solution

Due to the desire to rapidly change color output (possibly at the imager frame rate) the illuminator must be used in single shot mode, camera mode.  In this mode the illuminator will produce the desire exposure over some finite period of time.  The maximum pulse width will occur at the maximum exposure or the 100% exposure.  It will be necessary to check the maximum pulse width of the illuminator to insure that the integration time of the imager is sufficiently long at the 100% setting.  The duration of the exposure may not be determined until after the MakeItSo command has been issued.  It is necessary to provide an external trigger to initiate the exposure.

blSetIlluminatorConfigurationRegistryKey(“Software\Kodak\MyProduct”);	// Set the Registry Path

blSetIlluminatorCommunications();					// Enable communications

blSetIlluminatorLens(“None”);						// Do not simulate a lens in the optical path

const	double	dPie		= 3.141;

const	double	dColorTemp 	= 5500.0;

const 	double	dLevel		= 30.5;

const 	double 	dpmax		= 150.0;					// max scene highlights

const 	double	dpave		= 18.0;					// average scene highlights

const 	double	dFstop		= 4.0;					// f# of optical system

double	dExposureIndex;

// Solve equation 4 in the Camera Mode description for Exposure Index (EI).  We will calculate EI at the max code value

// hence CV = Cvmax and they cancel each other.

dExposureIndex = (40.0 * dpmax * dFstop * dFstop) / ( dLevel * dPie * dpave);

blSetIlluminatorModeSingleShot();						// Put Illuminator in Single Shot Mode

blSetIlluminatorIlluminantCIED(dColorTemp);				// Set up for Daylight 5500

blSetIlluminatorTestChart(“Kodak 64 Patch”)				// Desired test chart

// Enable camera mode and set camera mode parameters

blEnableIlluminatorCameraMode ();

blSetIlluminatorMaxSceneHighlightPercent(dpmax);

blSetIlluminatorAvgSceneReflectancePercent(dpave);

blSetIlluminatorExposureIndexOperation(dExposureIndex);

blSetIlluminatorLensFNumber(dFstop);

blSetIlluminatorMaxCodeValue(1023.0);

char		szError[iMAX_ERROR_LEN];

__int32	iNumbPatches;

__int32	iPatch;

iNumbPatches = iGetMaxNumbTestChartPatches ();

for(iPatch = 0; iPatch < iNumbPatches; iPatch++)

{

if(blSetIlluminatorTestChartPatchIndex(iPatch)) == FALSE)

{

blGetLastIlluminatorError(szError, iMAX_ERROR_LEN);

}

if(blIlluminatorCalcPulseWidthArrayToMemory (iPatch) == FALSE)

{

blGetLastIlluminatorError(szError, iMAX_ERROR_LEN);

}

}

blMakeItSo();								// Configure the Illuminator

// Download the maximum exposure to the illuminator so that we can determine the maximum pulse

// width.  This will allow us to determine the longest integration time that will be necessary.

blIlluminatorDownloadPulseWidthsFromMemory(i100_PERCENT);

m_dIllum100PerCentPulseWidth  = dGetCurrentSingleShotPulseWidth();

// Convert pulse width from usec to msec

m_dIllum100PerCentPulseWidth = m_dIllum100PerCentPulseWidth * 1000.0;

// We wish cycle through the color chart at 50% of full exposure

// We will first retrieve each color patch’s pulse widths from memory and load them into

// the illuminators firmware memory.  This allows for “hands off” sequencing through all

// of the color patches.  Each time the illuminator is strobed, the next color patch in the

// sequence will be displayed.

blDisableIlluminatorCameraMode ();

blSetScaleFactor(0.5);

// Load patches to illuminator

BOOL blLastPatch = FALSE;

for(iPatch = 0; iPatch < iNumbPatches; iPatch++)

{

// Download color patch to illuminator

if(iPatchNumb == (iNumbPatches - 1))

{

blLastPatch = TRUE;

}

if(blIlluminatorDownloadPulseWidthsFromMemoryToFirmwareMemory(iPatch, (BYTE)iPatchNumb, blLastPatch) == FALSE)

{

blGetLastIlluminatorError(szError, iMAX_ERROR_LEN);

}

}

for(iPatch = 0; iPatch < iNumbPatches; iPatch++)

{

// Every time the illuminator is triggered it will output the next color patch

// Capture and image and analyze

}

##### Index

Error! Bookmark not defined.blCalibrateLED, 29

blDisableIlluminator, 21

blDisableIlluminatorALC, 31

blDisableIlluminatorCameraMode, 10

blEnableIlluminator, 21

blEnableIlluminatorALC, 31

blEnableIlluminatorCameraMode, 10

blGetCurrentTestChartName, 23

blGetCurrentTestChartPatchName, 23

blGetIlluminantName, 19

blGetLastIlluminatorError, 28

blGetLensName, 26

blGetTestChartName, 23

blGetTestChartPatchName, 24

blIsIlluminatorModeContinuous, 8

blIsIlluminatorModeGated, 7

blIsIlluminatorModeRepetitive, 7

blIsIlluminatorModeSingleShot, 7

blIsIlluminatorModeSingleShotMemory, 8

blIsTestChartPatchIndexNeutral, 22

blPerformPhotoDiodeCalibration, 29

blSetIlluminatorALCAverage, 31

blSetIlluminatorALCRepetiveCycles, 31

blSetIlluminatorAvgSceneReflectancePercent, 3

blSetIlluminatorCommunications, 2

blSetIlluminatorConfigurationRegistryKey, 3

blSetIlluminatorExposureIndexOperation, 4

blSetIlluminatorExposureTime, 5

blSetIlluminatorGatedCycles, 16

blSetIlluminatorGatedFrequency, 11

blSetIlluminatorIlluminantCIEA, 17

blSetIlluminatorIlluminantCIEB, 17

blSetIlluminatorIlluminantCIEC, 17

blSetIlluminatorIlluminantCIED, 17

blSetIlluminatorIlluminantCIET, 18

blSetIlluminatorIlluminantCustom, 18

blSetIlluminatorIlluminantPlanckian, 18

blSetIlluminatorLens, 26

blSetIlluminatorLensFNumber, 4

blSetIlluminatorLuminousOutputLevel, 20

blSetIlluminatorMaxCodeValue, 4

blSetIlluminatorMaxSceneHighlightPercent, 3

blSetIlluminatorModeContinuous, 6

blSetIlluminatorModeGated, 6

blSetIlluminatorModeRepetitive, 6

blSetIlluminatorModeSingleShot, 6

blSetIlluminatorOperatingCodeValuePercent, 4

blSetIlluminatorRadiantOutputLevel, 20

blSetIlluminatorRepetitiveFrequency, 11

blSetIlluminatorSingleShotExposureValue, 13

blSetIlluminatorSingleShotPulseWidth, 13

blSetIlluminatorTestChart, 22

blSetIlluminatorTestChartPatchIndex, 22

blSetIlluminatorTestChartPatchName, 22

D

dGetCurrentRepetitiveExitPortLuminance, 14

dGetCurrentSingleShotExposureValue, 13

dGetCurrentSingleShotPulseWidth, 13

dGetIlluminatorGatedFrequency, 11

dGetIlluminatorRepetitiveFrequency, 11

dGetMaximumGatedExposureValue, 15

dGetMaximumSingleShotExposureValue, 14

dGetMaximumSingleShotPulseWidth, 14

dGetMinimumGatedExposureValue, 15

dGetMinimumSingleShotExposureValue, 15

dGetMinimumSingleShotPulseWidth, 14

I

iGetMaxIlluminantName, 19

iGetMaxLensName, 26

iGetMaxNumbIlluminants, 19

iGetMaxNumbLenses, 26

iGetMaxNumbTestChartPatches, 24

iGetMaxNumbTestCharts, 24

iGetMaxTestChartName, 24

iGetMaxTestChartPatchName, 25

Document Information:

Filename:	input.doc

Last saved:	1/26/2004 07:39:00 PM