X14 Product Specification

Eastman Kodak Part No. ######

Issue 1.0 - Sept. 10, 2003

by

FillFactory N.V.

Schaliënhoevedreef 20b

B-2800 Mechelen, Belgium

For

EASTMAN KODAK Company

Kodak Professional division

2400 Mt. Read Boulevard

Rochester, New York, 14650-3012

USA

Authors:	G. Meynants, W. Wuyts (FillFactory)

Doc. No.	2003-X14-GM-03

Distribution: 	S. Welk, S. Noble, M. Maurinus, G. Moberg, B. Pillman, J. White, R. Fish, D. Lutz, S. Harrington(Kodak Professional)

A. Alaerts, B. Dierickx, L. Hermans, S. Cos, L. De Mey (FillFactory)

Document history record

| Issue | Date | Description of changes |
| --- | --- | --- |
| 1.0 | 10-Sept-2003 | Initial release |

Review list

| Reviewer | Date |
| --- | --- |
| B. Dierickx |  |
| A. Alaerts |  |
| L. Hermans |  |

# Preamble

## Overview

This document is the specification of the X14 image sensor. This is a 14 Megapixel color CMOS active pixel image sensor. It is based on the high-fill factor active pixel sensor technology of FillFactory (US patent No. 6,225,670 and others).

## Main features

The main features of the image sensor are:

- 8x8 μm2 square pixels.

- 3048 x 4560 active pixels (13.9-mega-pixel)

- of which 3024 x 4536 pixels covered with bayer CFA

- 36 x 24 mm2 focal plane array (35 mm photographic film format).

- Frame rate: 3 frames/s (via 4 outputs).

- Max. 15 MHz pixel clock rate.

- Electronic rolling shutter.

- 4 parallel analog outputs.

- Optical dynamic range of 65 dB.

- On-chip fixed pattern noise correction.

TABLE OF CONTENT

1.	Preamble	3

1.1	Overview	3

1.2	Main features	3

2.	Architecture	6

2.1	Block diagram	6

2.2	Pixel architecture	7

3.	Specifications	8

3.1	General specifications	8

3.2	Electro-optical specifications	8

3.3	Spectral response curve	10

3.4	Electrical specifications	11

3.4.1	Output stage	11

3.4.2	Absolute maximum ratings	13

3.4.3	Recommended operating DC and logic level specifications	13

3.4.4	Environmental specifications	13

3.4.5	Bias currents and references	14

3.4.6	Handling	14

4.	Image quality acceptance criteria	16

5.	Operation	20

5.1	Pixel array	20

5.2	Readout and sub-sampling modes	21

5.3	Stitch pattern configuration	23

5.4	Sensor output stage	24

5.4.1	Amplifier specifications	24

5.4.2	Output amplifier crossbar switch (multiplexer)	24

5.5	Sensor readout timing diagrams	26

5.5.1	Row sequencer	26

5.5.2	Timing pulse pattern for readout of a pixel	30

5.5.3	Fast frame reset timing diagram	31

5.6	SPI register	33

5.6.1	SPI Interface architecture	33

5.6.2	SPI register definition	34

6.	Pin configuration	36

7.	Geometry & mechanical	39

7.1	Die geometry	39

7.2	Pin number assignment	40

7.3	Cover glass	41

8.	Appendix 1: drawings	42

# Architecture

## Block diagram

The basic architecture of the sensor is shown in Figure 1.

Figure 1: imager architecture

The Y shift registers point at a row of the imager array. This row is selected and/or reset by the row drivers. There are 2 Y shift registers: one points at the row that is read out and the second one points at the row to be reset. The second pointer may lead the first pointer by a specific number of rows. In that case, the time difference between both pointers is the integration time. Alternatively, both shift registers be point at the same row for reset and readout for a faster reset sequence. Readout is destructive: when the row is read out, it is also reset in order to do double sampling for fixed pattern noise reduction.

The pixel array of X14 contains 4536 x 3024 active pixels covered wit ha bayer CFA pattern and 24 additional columns and rows, which can also be addressed (see Figure 2). These columns and rows are covered with CFA test patterns and are not intended for imaging.

The column amplifiers read out the pixel information and perform the double sampling operation. They also multiplex the signals on the readout busses, which are buffered by the output amplifiers.

The shift registers can be configured for various sub-sampling modes. The output amplifiers can be individually powered down. And some other extra functions are foreseen. These options are configurable via a serial input port.

## Pixel architecture

Figure 2: pixel schematic

The pixel is a classic 3-transistor active pixel. The photodiode is a high-fill factor n-well/p-substrate diode. Separate power supplies are foreseen: general power supply for the analog image core (VDD), power supply for the reset line drivers (VDDR) and a separate power supply for the pixel itself (VDDARRAY).

The color filter array is a “bayer” configuration. The first active pixel of the image is pixel (0,24) and is green of a green-red row.

# Specifications

## General specifications

Table 1:  general specifications

| Parameter | Value |
| --- | --- |
| Pixel architecture | 3 transistor active pixel |
| Technology | 0.35 µm 2 poly 3 metal CMOS |
| Pixel size | 8 x 8 μm2 |
| Resolution | 3048 x 4560 |
| Power supply | 3.3V |
| Shutter type | Electronic rolling shutter |
| Pixel rate | 12.5 MHz nominal |
| Power dissipation | 176 mW 53 mA |

## Electro-optical specifications

Table 2 lists the electro-optical specifications. All parameters are set using the default settings (see recommended operating conditions), unless otherwise specified.

Table 2: electro-optical specifications.

| Parameter | Average Value | Remark |
| --- | --- | --- |
| Effective conversion gain | 18.5 μV/e- | See note 1. |
| Spectral response * fill factor (monochrome) | 0.2 A/W (peak) |  |
| Peak Q.E. * fill factor (monochrome) | 40% | Between 500 and 700 nm. |
| Full Well charge | 64865 electrons | See note 1. |
| Linear range | 90 % of full well charge | Linearity definition: < 3% deviation from straight line through zero point. |
| Temporal noise (short exposure times) | 35 electrons | Dominated by kTC noise in the dark at short integration times. |
| Dynamic range | 1871:1 (65.4 dB) | See note 1. |
| Linear dynamic range | 1688:1 (64.5 dB) | See note 1. 3% deviation. |
| Average dark current | 55 pA/cm2 | Average value At 24 °C lab temperature. |
| Dark current signal | 223 electrons/s 4.13 mV/s (average) Limit TBD (target : 15 mV/s @ 30 °C, i.e. 10 mV at 24 °C) | Average value of center pixels At 24 °C lab temperature (measured at wafer level) |
| Dark current temperature coefficient | Doubles every 9.3 °C |  |
| MTF at Nyquist | 0.55 in X 0.57 in Y | Measured at 600 nm. |
| Fixed pattern noise (local) | 0.11 % Vsat RMS | Average value of RMS variation on local 32 x 32 pixel windows at short exposure time (11ms). |
| Fixed pattern noise (global) | 0.15% Vsat RMS | Calculated on all 4000 x 3500 pixels; short exposure time (11 ms) |
| DSNU (local) | 0.39% Vsat RMS | Local; 357 ms exposure time |
| DSNU (global) | 0.75% Vsat RMS | Global, 357 ms exposure time |
| PRNU (low frequency) | 1.12 % RMS of signal | RMS of mean of 96 x 96 tiles |
| PRNU (high frequency) | 1.22 % RMS of signal | Average RMS of 96 x 96 tiles |

Notes:

- Settings: VDD=3.3 V, VDDR =4V and VDD_ARRAY = 3V.

## Spectral response curve

Figure 3: Spectral response curve (monochrome)

Figure 3: quantum efficiency * fill factor measured on monochrome X14 (and C14 for comparison)

Figure 4: quantum efficiency * fill factor measured on X14 (including S8612 cover glass)

## Electrical specifications

### Output stage

Unity gain buffers are implemented as output amplifiers. These amplifiers can directly be DC coupled to the analog-digital converter or coupled to an external programmable gain amplifier.

The (dark reference) offset of the output signal is adjustable between 1.7 and 3 V. The amplifier output signal is negative going with increasing light levels, with a max. amplitude of 1.2 V (at 4V reset voltage, in hard reset mode). Table 3 summarizes the electrical specifications.

Table 3: Electrical specifications

| Parameter | Value | Remark |
| --- | --- | --- |
| Nominal power supply | 3.3 V | Max. 3.6 V |
| Frame rate | 3.25 frames/s (1) | (1): 4 parallel analog outputs. (excluded readout of “reset black” pixels). |
| Output signal amplitude | 1.2 V | Negative signal polarity (lower signal for increasing light levels). |
| Output signal range | 0.5 – 3 V | Offset adjustable with DARKREF input pin |
| Slew rate | To be measured | Can be tuned by resistor connected to OBIAS pin |
| Settling time (stable output within 500 μV from final value) | To be measured |  |

Notes on analog video signal and output amplifier specifications:

- Video polarity: the video signal is negative going with increasing light level.

- Signal offset: the analog offset of the video signal is settable by an external DC bias (pin 12 DARKREF). The settable range is between 1.7 and 3 V, with 2.65 V being the nominal expected set point. The output range (including 1.2 V video signal) is thus between 3 V and 0.5V.

- Power control: the output amplifiers can be switched between an “operating” mode and a “standby” mode via the serial port of the imager (see SPI register configuration). Power reduces from 53 mA to 20 mA (when the device is being clocked). Stopping the CLK_X clock further reduces the power consumption.

- Coupling: the X14 can be DC or AC coupled to the AD converter.

### Absolute maximum ratings

Table 4: Absolute maximum ratings

| Symbol | Parameter | Value | Unit |
| --- | --- | --- | --- |
| VDC | DC power supply voltage (except VDDR) | -0.5 to +3.6 | V |
| VIN | DC input voltage | -0.5 to VDC + 0.5 | V |
| VOUT | DC output voltage | -0.5 to VDC + 0.5 | V |
| I | DC current per pin; any single input or output. | ± 50 | mA |
| Idd | Max. input current | 80 | mA |

Note: Absolute Ratings are those values beyond which damage to the device may occur.

### Recommended operating DC and logic level specifications

Table 5: Recommended operating conditions

| Symbol | Parameter | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| VDD | Nominal power supply. | 3 | 3.3 | 3.6 | V |
| VDDR | Reset power supply level. | 3 | 4 | 4.4 | V |
| VDD_ARRAY | Pixel supply level. |  | 3 | 3.3 | V |
| DARKREF | Dark reference offset level. | 1.7 | 2.65 | 3 | V |
| GNDAB | Anti-blooming ground level. | 0 | 0 | 1 | V |
| VOUT | Analog output level. | 0.5 |  | 3 | V |
| VIH | Logic input high level. | 2.5 |  | 3.3 | V |
| VIL | Logic input low level. | 0 |  | 1 | V |

### Start-up and idle behaviour

Unlike C14, there is no high-current state at start-up. This is prevented by an SPI bit that must be turned on before the column amplifiers are powered up via SPI bit 15. Before powering on this bit, the CLK_X clock must start to run. It appears that the SPI bits are all set to zero at power up (but no test is done to guarantee that on each device).

Unlike C14, there is no mode in which the chip will start to consume more power after a certain idle time.

### Environmental specifications

Storage temperature:

-10 ºC @ 15% RH (RH = relative humidity)
	66 ºC @ 15% RH
	38 ºC @ 86% RH

Operating temperature:

0 ºC @ 15% RH (RH = relative humidity)
	50 ºC @ 15% RH
	38 ºC @ 86% RH

Altitude: max. 8000 feet

### Bias currents and references

Table 6: bias input currents

| Pin number | Pin name | Connection | Input current | Pin voltage |
| --- | --- | --- | --- | --- |
| 1 | OBIAS | 10 kΩ to VDD | 179 uA | 1.51 V |
| 36 | CBIAS | 22 kΩ to VDD | 91 uA | 1.29 V |
| 37 | PCBIAS | 22 kΩ to VDD | 91 uA | 1.29 V |
| 48 | XBIAS | 10 kΩ to VDD | 181uA | 1.49 V |
| 49 | ABIAS | Gnd or open (or 10M to VDD) |  | 0.8 V in case of 10 M |

Tolerance on bias reference voltages: +/- 150 mV

### Handling

#### ESD

Though not as sensitive as CCD sensors, the X14 is vulnerable to ESD like other standard CMOS devices. Take into account standard ESD procedures when manipulating the device.

  - Always discharge static electricity by grounding the human body and the instrument to be used. To ground the human body, provide a resistance of 1 MOhm between the human body and the ground to be on the safe side.

  - When directly handling the device with the fingers, hold the part without the leads and do not touch any lead.

  - To avoid generating static electricity:

    - do not scrub the glass surface with cloth or plastic

    - do not attach any tape or labels

    - do not clean the glass surface with dust-cleaning tape

  - When storing or transporting the device, put it in a container of conductive material.

#### Dust and Contamination

Dust or contamination of the glass surface could deteriorate the output characteristics or cause a scar. In order to minimize dust or contamination on the glass surface, take the following precautions:

- handle the device in a clean environment such as a cleaned booth (the cleanliness should be, if possible, class 100)

- Do not touch the glass surface with the fingers.

- Use gloves to manipulate the device

#### Soldering

Soldering should be manually performed with 5 seconds at 350 C maximum at the tip of the soldering iron

Avoid mechanical stress when mounting the device.

# Image quality acceptance criteria

Table 7 lists the acceptance criteria for the device. Devices will be tested with these limits after assembly of the chip. Dark current acceptance will be tested at wafer level (because of the better control over the die temperature). The table also lists the parameter as named in the device/wafer tests and the test method.

The limits will be reviewed at regular times. Limits listed here today (Sept. 10) are based upon limited statistics from the first batch of sensors only and upon devices tested under different conditions from the final configuration (e.g. higher power supply).

A first review of the limits is planned after test of the next 30 image sensors, and again after the next 50, 200 and 500 produced image sensors. Limits are determined based upon the statistics on the “supposed good” devices. The average value is listed as the typical value for the parameter, the min and max limit are determined as a 6-sigma variation on the measured data. Some parameters (like dark current and maybe also DSNU because it relates to dark current) depend most on the wafer lot (and are relively stable within a lot). Statistics to determine the limits should include these wafer lot to wafer lot variations.

Table 7: X14 Purchaise specification

| # | Test /parameter | Specification / limits. | Test Method / Description |
| --- | --- | --- | --- |
| 1 | Saturation (= Vsat) Vsweep | Min. 1.00 V Typ. 1.16 V Max. 1.32 V | Uniformly illuminate sensor to 2x above Vsat of lowest RGB channel; measure average value of dark image and average value of 2x Vsat image and subtract both averages |
| 2 | Max Device Current ITOT_OP | Min. 40 mA Typ. 53 mA Max. 80 mA | No illumination @ 250 ms integration time. (these limits are wider than 6 sigma) |
| 3 | DSNU + FPN DSNU#rmean DSNU#pp | average and p/p of tile-RMS-es Average of tile RMS (DSNUrmean) typ. 3 mV max. ? (6 mV TBD) p/p of tile RMS (DSNUrpp) typ. 1.7 mV Max. ? (3.6 mV TBD) | No illumination @ 250ms integration time ; measure all 96 x 96 tiles in image Measured separately for each output channel TBD values only determined after first lots and fixed when a stable value is reached |
| 4 | Reset black test BREF#pp | Peak-to-peak variation of reset black area (64 pixels/channel) Min. 0 Typ. 5 mV Max. 30 mV | Readout Reset Black values (using Reset Black timing) of the first 64 pixels of each row for each output. Calculate peak-to-peak of row averages; measure separately on each output. |
| 5 | Dark Current DC | Min. 0 Typ. 7.5 mV/s @ 30 ºC Max. (15 mV/s ?) TBD @ 30 ºC (suggested 15 mV/s @ 30 ºC but more lot statistics required) | No illumination; measured by average dark signal difference between sub-frames with long (1.310 s) and short (250 ms) integration time. Sub-frame: between (1500,1000) and (2500,1700) (700,000 pixels in total) Measure on wafer at stabilized die temperature (30 ºC) |
| 6 | PRNU Prnu#<col>-mpp | At this moment calculated as pTp of tile averages (not related to center tile but to average of tiles) Limits (from statistics on first sensors) Min: 0 Typ: 5.3% pTp of signal Max: 11% pTp of signal | Uniform illumination at 70% Vsat. ; 250ms integration time ; 96 x 96 tiles; calculate average and p/p of tile averages after dark frame subtraction. 10% p-p in relation to center tile average; each color Gr,Gb,R,B separate |
| 7 | Sensitivity | Min. ? Typ: ? Max. ? Question : should we include this test (never implemented in C14 – tester stability can be an issue) | Uniform illumination at 70% of VSAT; 250 ms integration time. Calculate average value per color channel after dark frame subtraction |
| 8 | Clusters / correctable pixel test nTotPixNoCorr | Max. 0 non correctable pixels All defect pixels must be correctable. | start from defect pixel map of tests 9 and 10. The non-defective neighbors must be within 2 pixels horizontally, vertically or diagonally of the same color. A defect pixel must have either: - two non-defect neighbors of the same color (R, Gb, Gr, B), out of the four horizontal and vertical neighbors - two diagonal non-defect neighbors of the same color, both located on the same diagonal crossing the pixel - Gr pixels can be corrected with either two Gb diagonal neighbors or with two Gr diagonal neighbors |
| 9 | Defective Pixels nDefectSingle | a defective pixel is one that has more than +10% Vsat/-15% Vsat deviation from the local average; max total point defects + defect columns and rows: 3000 pixels Add to defect map | Uniform illumination at 70% Vsat. Subtract dark frame Local average calculated in 96x96 tiles Defective pixels added to defect pixel map Count defect map size (# pixels + # columns + # rows) |
| 10 | Bright Point Defects NbrTotPIxNo- Corr | 10% Vsat above threshold of average local dark floor (96 x 96 tiles) at 25 ºC (14% Vsat at 30 ºC) Add to defect map | No illumination @ 250 ms integration time. Separate dark floor calculated for each output channel Points added to defect pixel map Wafer test is at 30 degrees, bright pixel threshold is raised to 14% for wafer test. |
| 11 | Row/Column Defects NCol5+ NCol# NRow5+ NRow# NColTot NRowTot MinDistBadCol MinDistBad- Row | 2% Vsat deviation from local average for each color under test, either G&B or G&R; Max. 50 total columns + rows ; 5 double columns allowed 5 double rows allowed Min. 3 good columns in between any pair of bad columns | start from the defect pixel maps in test 9 + test 10. Local average: 96 columns moving neighbourhood A bad row/column is either: 1. has more than 300 defect pixels accodring to test 9+10 2. of which the average signal deviates more than 2% Vsat from its neighbors in the 70% image (after dark frame subtraction). For the 10 outer columns and rows 4.5% Vsat is used as limit. 3. a shorted columns which is detected via a specific test with colored light (not detected via other means) |
| 12 | Coverglass Dig/Scratch NTotPixNoCorr | Max. 0 non correctable pixels. | Uniform illumination at 70% Vsat with high F-number F11. Test sensor for defective pixels. Defects on cover glass will generate defect pixels in defect map. All defect pixels must be correctable. Same tool is used for cover glass inspection in production flow. |
| Measurement conditions: |  |  |  |
| 1. Illumination is 5500K daylight |  |  |  |
| 2. Temperature is 25ºC (logged during the test program) and 30 ºC on wafer. Dark current limit is set at 30 ºC; bright point spec is 10% at 25ºC and 14% at 30ºC. |  |  |  |
| 3. Illumination levels are referenced to the largest of the 3 RGB channels (green) |  |  |  |
| 4. The testing tiles are 96 x 96 pixels (a multiple of 24 which is the sub sampling kernel) |  |  |  |
| 5. Note that for most tests, Gr and Gb are treated separately due to GNU |  |  |  |
| 8. Definition of operation conditions: 4 outputs, SPI mode 33342 (no sub-sampling) VDD = 3.3 V; VDD_ARRAY = 3.0 V; VDDR = 4.0 V; DARKREF = 2.65 V; GND_AB = 0 V Readout mode: single-shot mode (no rolling shutter) : reset frame, integrate light, read out reset frame: 76 ms (dark) capture: 250 ms (illuminated) read out time: 330 ms @ 12.5 MHz pixel rate (dark during readout) The first row has an effective integration time of (76 ms + integration time). The last row has an integration time of (330 ms + integration time) |  |  |  |
| 9. All 3024 x 4536 pixels should be tested. Cluster/correctable pixel test not performed on first and last row/column (because lack of neighbours) |  |  |  |

Every device passes a test program that tests the specifications listed in the above table. The device also passes a set of electrical and functional tests:

  - contact test

  - tests of output amplifiers

  - cross-bar switch test

  - tests for specific failure modes (e.g .row streaking)

# Operation

## Pixel array

Figure 5 shows the scan direction and the location of pixel (0,0), the 24 extra rows and columns and the position of pin 1 and the analog outputs relative to the pixel array.

Figure 5: Location of the 24 additional columns and rows, scan direction of the array and location of the analog outputs. Sensor shown as mounted in a camera in landscape format.

## Readout and sub-sampling modes

The sub-sampling modes available on the X14 are summarized in Table 7.

Table 8: Sub-sample modes

| Subsampling modes programmed into SPI register |  |  |
| --- | --- | --- |
| X shift register subsampling settings |  |  |
| Bitcode | Mode | Use |
| 000 001 010 | 1:1 Full resolution (all columns) | Full resolution (4 outputs) 4:1 subsampling |
| 011 | 24:1 Select 4 columns/ skip 20 | 24:1 subsampling ( 2 outputs) |
| 100 | 8:1 Select 4 columns / skip 4s | 8:1 subsampling (2 outputs) |
| 101 | 12:1 Select 4 columns / skip 8 | 12:1 subsampling (2 outputs) |
| Y shift register subsampling settings |  |  |
| Bitcode | Mode | Use |
| 000 010 100 | 4:1 Select 2 rows / skip 2 | 4:1 subsampling |
| 001 | 1:1 Full resolution (all rows) | Full resolution |
| 011 | 6:1 Select 2 rows / skip 4 | 6:1 subsampling |
| 101 | 12:1 Select 2 rows / skip 10 | 12:1 subsampling |

Each mode is selected independently for the X and Y shift registers. The sub-sampling mode is configured via the serial input port of the chip. The Y and X shift registers have some different sub-sampling modes, due to constraints in the design of the chip.

The baseline full resolution operation mode uses 4 outputs to read out the entire image. 4 consecutive pixels of a row are put in parallel on the 4 parallel outputs.

Sub-sampling is implemented by a shift register with hard-coded sub-sample modes. Depending on the selected mode, the shift register skips the required number of pixels when shifting the row or column pointer.

The X shift register always selects 4 consecutive columns in parallel. Sub sampling in X can be done by activating one of the modes wherein a multiple of 4 consecutive columns is skipped on a CLK_X pulse. The Y shift register selects a single row. It will consecutively select 2 adjacent rows and then skip an amount of rows set by the sub sample mode. This implementation is chosen for easy sub sampling of color images through a 2-channel readout. In this way color data from 2x2 pixels is made available in all sub sample modes. Note that for 2 or 1 channel readout, the not-used output amplifiers can be powered down through the SPI shift register.

Rows can also be skipped by extra CLK_Y pulses. It is not required to apply additional control pulses to rows that are skipped. This is a way to implement extra sub-sampling schemes. For example, to support the 24:1 X shift register mode also vertically, the Y shift register can be set to the 12:1 mode and an additional CLK_Y pulse needs to be given at the start of each row.

Table 8 lists the frame rates of X14 in various sub-sample modes with only one output. The row blanking time (dead time between readout of successive rows) has been set to 17.5 μs.

Table 9: frame rates and resolution of X14 in various sub-sampling modes

| Ratio | #outputs | Image Resolution | Frame rate frames/s | Frame readout time [s] |
| --- | --- | --- | --- | --- |
| 1:1 | 4 | 3024 x 4536 | 3.25 | 0.308 |
| 4:1 | 1 | 756 x 1134 | 12.99 | 0.077 |
| 8:1 | 1 | 378 x 567 | 41.30 | 0.024 |
| 12:1 | 1 | 252 x 378 | 77.13 | 0.013 |

Note that the 24 additional columns and rows (see Figure 2) do not sub sample (foreseen for an optical black).

## Stitch pattern configuration

Figure 6: stitch pattern configuration

Figure 6 shows the configuration of the stitch pattern. It might be that the rows at the borders of each block have a slightly different sentitivity or that some block-to-block sensitivity variations can be observed.

## Sensor output stage

### Amplifier specifications

Unity gain buffers are implemented as output amplifiers. Paragraph 3.4 lists the specifications of the output amplifier stage.

### Output amplifier crossbar switch (multiplexer)

A crossbar switch is available that routes the green pixels always to the same output (for a color device). The switch can be controlled automatically (with a toggle on every CLK_Y rising edge) or manually (through the SPI register).

Figure 6 shows how it works. A pulse on SYNC_Y resets the crossbar switch. The initial state after reset of the switchboard is read from the SPI control register. When the automatic toggling of the switchboard is enabled, it toggles on every rising edge of the CLK_Y clock. Separate pins are used for the SYNC_Y and CLK_Y signals on the crossbar logic these pins can be connected to the SYNC_YL and CLK_YL pins of the shift register that is used for readout.

Figure 4: output amplifier crossbar switch

## Sensor readout timing diagrams

### Row sequencer

The row sequencer controls pulses to be given at the start of each new line. Figure 7 shows the timing diagram for this sequence.

The signals to be controlled at each row are:

- CLK_YL and CLK_YR: these are the clocks of the YL and YR shift register. They can be driven by the same signals and at a continuous frequency. At every rising edge, a new row is being selected.

- SELECT: this signal selects the line that is currently sampled. It is important that PC and SELECT are never active together.

- PC: an initialization pulse that needs to be given after the selection of a new row.

- SHS (Sample & Hold pixel Signal): this signal controls the track & hold circuits in the column amplifiers. It is used to sample the pixel signal in the columns. (0=track ; 1=hold)

- RESET: this pulse resets the pixels of the row that is currently being selected. In rolling shutter mode, the RESET signal is pulsed a second time to reset the row selected by the YR shift register. For “reset black” dark reference signals, the reset pulse can be pulsed also during the first PC pulse. Normally, RESET and PC have a rising edge at the same position. The falling edge of RESET lags behind the falling PC edge.

- SHR (Sample & Hold pixel Reset level): this signal controls another track & hold circuit in the column amplifiers. It is used to sample the pixel reset level in the columns (for double sampling). (0=track ; 1=hold)

- SYL (Select YL register): Selects the YL shift register to drive the reset line of the pixel array

- SYR (Select YR register): Selects the YR shift register to drive the reset line of the pixel array. For rolling shutter applications, SYL and SYR are complimentary. In full frame readout, both registers may be selected together, only if it is guaranteed that both shift registers point to the same row. This can reduce the row blanking time.

- SYNC_YR and SYNC_YL: Synchronization pulse for the YR and YL shift registers. The SYNC_YR/SYNC_YL signal is clocked in during a rising edge on CLK_YR/CLK_YL and resets the YR/YL shift register to the first row. Both pulses are pulsed only once each frame. The exact pulsing scheme depends on the mode of use (full frame/ rolling shutter). A 200 ns set-up time applies. See further.

Figure 5: row sequencer timing diagram

- SYNC_X: resets the column pointer to the first row. This has to be done before the end of the first PC pulse, in case when the previous line has not been read out completely.

Figure 5 shows the basic timing diagram of the X14 image sensor and Table 10 shows the timing specifications of the clocking scheme.

Table 10: Timing specifications

| Symbol | Min | Typ | Description |
| --- | --- | --- | --- |
| a |  | 6.8 us | Delay between falling edge on PC and rising edge on SHS/SHR. Duration of SHS/SHR pulse. |
| b | 200 ns | 600 ns | Min. SYNC setup times. SYNC_Y is clocked in on rising edge on CLK_Y. SYNC_Y pulse must overlap CLK_Y by one clock period. Setup times of 200 ns apply after SYNC edges. Within this setup time no rising CLK edge may occur. |
| c |  | 2.7 us | Duration of PC pulse. |
| d |  | 4 us | Duration of RESET pulse. |
| e | d + 2 * CLK | 0.5 us | SYL and SYR pulses must overlap second RESET pulse at both sides by one clock cycle. |
| f | 0us |  | Delay between rising edge on CLK_YR and falling edge on SELECT. |
| g | 1 CLK | 0.1 us | Delay between rising edge on SHS and falling edge on SELECT. |
| k | 1 CLK | 0.1 us | Delay between rising edge on SHR and rising edge on SYR |
| m |  | 17.5 us | Minimal total idle time between readout of two rows (vertical interval time). |
| o |  | 1.4 us | Delay between falling edge SELECT and PC. |
| p |  | 5.4 us | Total SELECT pulse duration (low period). |
| q |  | 1.3 us | Delay between rising edge on PC and rising edge on SELECT. |
| r |  | 6.6 us | Delay between rising edge on SELECT and rising edge on SHS/SHR. |

Notes:

- CLK = one clock period of the master clock, shortest system time period available.

- Red items apply each row; green items apply once each frame.  Blue is for the additional reset for reset black pixels.

In the above timing diagram, the YR shift register is used for the electronic shutter. The CLK_YR is driven identically as CLK_YL. The SYNC_YR pulse leads the SYNC_YL pulse by a given number of rows. Relative to the row timing, both SYNC pulses are given at the same time position.

SYNC_YR and SYNC_YL are only pulsed once each frame, SYNC_YL is pulsed when the first row will be read out and SYNC_YR is pulsed for the electronic shutter at the appropriate moment in time.

This timing assumes that the registers that control the sub-sampling modes have been loaded in advance (through the SPI interface), before the pulse on SYNC_YL or SYNC_YR.

The second reset pulse and the pulses on SYL and SYR (all pulses drawn in red) are only applied when the rolling electronic shutter is used. For full frame integration, these pulses are skipped.

The SYNC_Y pulse is also used to initialise the switchboard (output multiplexer). This is also done by a synchronous reset on the rising edge of CLK_Y. Normally the switchboard is controlled by the shift register used for readout (this is the YL shift register). This means that pin SYNC_Y can be connected to SYNC_YL, and pin CLK_Y can be connected to CLK_YL.

The additional RESET BLACK pulse (indicated in blue in Figure 5) can be given to make one or more lines black. This can be useful to generate a dark reference signal.

### Timing pulse pattern for readout of a pixel

Figure 6 shows the timing diagram to preset (sync) the X shift register, read out the image row, and analog-digital conversion. There are 3 tasks:

Figure 6: Pixel row read-out timing

- preset the X shift register : apply a low level to SYNC_X during a rising edge on CLK_X at the start of a new row

- readout of the image row : pulse CLK_X

- analog-digital conversion :  clock the ADC

The SYNC pulses perform a synchronous reset of the shift registers to the first row/column on a rising edge on CLK. This is identical for all shift registers (YR, YL and X).

Important : 	The SYNC_X signal has a set-up time Ts of 150 ns. For the YR and YS shift registers, the set-up time is 200 ns. CLK_X must be stable at least during this set-up time.

In case where a partial row readout has been performed, 2 CLK_X pulses (with SYNC_X = LOW) are required to fully deselect the column where the X pointer has been stopped. A single CLK_X will leave the column partially selected, which will then have a different response when read out in the next row. When full row readout has been performed, the last column will be fully deselected by a single CLK_X pulse (with SYNC_X=LOW). The X-register is reset by a single CLK_X pulse (with SYNC_X=LOW). In case of partial row readout, the SYNC_X pulse has to be given before the sample pulses (SHR and SHS) of the row sampling process, in order to avoid a different response of the last column of the previous window.

For the X shift register, the analog signal is delayed by 2 clock periods before it becomes available at the output (due to internal processing of the signal in the columns and output amplifier). The figure gives an example of an ADC clock for an ADC that samples on the rising edge.

### Fast frame reset timing diagram

Figure 9 shows the reset timing for a fast frame reset.

SYL and SYR can be kept both high to make the reset mechanism faster and reduce propagation delays. PC, SHS, SHR can be kept high since they don’t interact with the pixel reset mechanism. Table 11 lists timing specifications for RESET, CLK_Y and SELECT.

Table 11: timing specifications for fast reset (preliminary)

| Symbol | Typical | Description |
| --- | --- | --- |
| a | 0 us | Delay between rising CLK_Y edge and Reset. |
| b | 4 us | Reset pulse width. |
| c | 0 | Reset hold time. |
| d | 1.6 us | Select pulse width. |
| e | 1 us | Setup hold time. CONSTRAINT : a + e > 1 us due to propagation delay on pixel select line. |

Figure 7: fast frame reset timing

## SPI register

### SPI Interface architecture

The elementary unit cell of the serial to parallel interface consists of two D-flip-flops. The architecture is shown in Figure 10. 16 of these cells connected in parallel, having a common /CS and SCLK form the entire uploadable parameter block, where Din is connected to Dout of the next cell. The uploaded settings are applied to the sensor on the rising edge of signal /CS.

Figure 8:  Uploadable parameter part

Table 12: Timing requirements serial-parallel interface

| Parameter | value |
| --- | --- |
| Tsclk | 100ns |
| Ts | 50ns |
| Th | 50ns |

### SPI register definition

Sensor parameters can be serially uploaded inside the sensor at the start of a frame. The parameters are :

- Sub-sampling modes for X and Y shift registers (3-bit code for 6 sub-sampling modes)

- Power control of the output amplifiers, column amps and pixel array. Each amplifier can be individually powered up/down

- Output crossbar switch control bits. The crossbar switch is used to route the green pixels to the same output amplifiers at all time. A first bit controls the crossbar. When a second bit is set, the first bit will toggle on every CLK_Y edge in order to automatically route the green pixels of the bayer filter pattern.

The code is uploaded serially as a 16-bit word (LSB uploaded first). Table 13 lists the register definition.

Table 13: serial sensor parameters register bit definitions

| BIT | Description |
| --- | --- |
| 0 (LSB) | set to zero (0) |
| 1 | 1= power on sensor array ; 0= power down |
| 2 | 1 = power up output amplifier 4; 0 = power down |
| 3 | 1 = power up output amplifier 3; 0 = power down |
| 4 | 1 = power up output amplifier 2; 0 = power down |
| 5 | 1 = power up output amplifier 1; 0 = power down |
| 6 | 3 –bit code for sub-sampling mode of X shift register: 000 = full resolution 011 = select 4, skip 20 001 = full resolution 100 = select 4, skip 4 010 = full resolution 101 = select 4, skip 8 |
| 7 |  |
| 8 |  |
| 9 | 3 –bit code for sub-sampling mode of Y shift registers: 000 = select 2, skip 2 011 = select 2, skip 4 001 = full resolution 100 = select 2, skip 2 010 = select 2, skip 2 101 = select 2, skip 2 |
| 10 |  |
| 11 |  |
| 12 | Crossbar switch (output multiplexer) control bit initial value. This initial value is clocked into the crossbar switch at a SYNC_YR rising edge pulse (when the array pointers jump back to row 1). The crossbar switch control bit selects the correspondence between multiplexer busses and output amplifiers. Bus-to-output correspondence is according to the following table: Bus when bit set to 0 when bit set to 1 1 output 1 output 2 2 output 2 output 1 3 (4 outputs) output 3 output 4 4 (4 outputs) output 4 output 3 |
| 13 | 1 = Toggle crossbar switch control bit on every odd/even line. In order to let green pixels always use the same output amplifier automatically, this bit must be set to 1. On every CLK_Y rising edge (when a new row is selected), the crossbar switch control bit will toggle. Initial value (after SYNC_Y) is set by bit 12. |
| 14 | Not used. |
| 15 (MSB) | 1=Power up sensor array ; 0 = Power down |

3 pins are used for the serial data interface. This interface converts the serial data into an (internal) parallel data bus (Serial-Parallel Interface or SPI). The control lines are:

- DATA : the data input. LSB is clocked in first.

- CLK : clock, on each rising edge, the value of DATA is clocked in

- CS : chip select, a rising edge on CS loads the parallelized data into the on-chip register.

The initial state of the register is undefined. However, no state exists that destroys the device. It appears that the start-up value is 0 in normal cases (but no test is done to verify this on each device). There is also no mode wherein the current consumption is high (unlike C14 during start-up).

The default SPI code for a full resolution readout is 33342 (decimal) or 1000 0010 0011 1110  (power on of all amplifiers and column amplifiers, no subsampling, no crossbar switch).

# Pin configuration

Table 14 lists the pin configuration of X14. Figure 9 shows the assignment of pin numbers on the package.

Table 14: Pin list

| Pin nr. | Name | Function | Comment |
| --- | --- | --- | --- |
| 1 | OBIAS | Bias current output amplifiers. | Connect with 10k to VDD and decouple with 100 nF to GND. |
| 2 | GND | Ground for output 3. |  |
| 3 | OUT3 | Output 3. |  |
| 4 | GND | Ground for output 4. |  |
| 5 | OUT4 | Output 4. |  |
| 6 | VDD | Power supply. | Nominal 3.3 V |
| 7 | GND | Ground. | 0 V |
| 8 | OUT2 | Output 2. |  |
| 9 | GND | Ground for output 2. |  |
| 10 | OUT1 | Output 1. |  |
| 11 | GND | Ground for output 1. |  |
| 12 | DARKREF | Offset level of output signal. | Typ. 2.6 V. min. 1.7 V max. 3 V |
| 13 | TEMP1 | Temperature sensor. Located near the output amplifiers (pixel 4536, 0) near the stitch line). | Any voltage above GND forward biases the diode. Connect to GND if not used. |
| 14 | PHDIODE | Photodiode output. Yields the equivalent photocurrent of 250 x 50 pixels. Diode is located right under the pad. | Reverse biased by any voltage above GND Connect to GND if not used. |
| 15 | CLK_Y | Y clock for switchboard. | Clocks on rising edge Connect to CLK_YL (or drive identically) |
| 16 | SYNC_Y | Y SYNC pulse for switchboard. | Low active: synchronous sync on rising edge of CLK_Y Connect to SYNC_YL (or drive identically) |
| 17 | TEMP2 | Temperature sensor. Located near pixel (24,0). | Any voltage above GND forward biases the diode. Connect to GND if not used. |
| 18 | GNDAB | Anti-blooming reference level (=pin 33). | Typ. 0 V. Set to 1.5 V for improved anti-blooming. |
| 19 | GND | Ground. | 0 V |
| 20 | VDD | Power supply. | Nominal 3.3 V |
| 21 | VDDR | Power supply for reset line drivers (= pin 30) | Nominal 4 V Connected on-chip to pin 30 |
| 22 | CLK_YR | Clock of YR shift register. | Shifts on rising edge. |
| 23 | SYR | Activate YR shift register for driving of reset and select line of pixel array. | High active. Exact pulsing pattern see timing diagram. Both SYR = 1 and SYL = 1 is not allowed, except when the same row is selected! |
| 24 | SYNC_YR | Sets the YR shift register to row 1. | Low active. Synchronous sync on rising edge of CLK_YR 200 ns setup time |
| 25 | VDDARRAY | Pixel array power supply (= pin 26). | 3 V |
| 26 | VDDARRAY | Pixel array power supply (= pin 25). | 3 V |
| 27 | SYNC_YL | Sets the YL shift register to row 1. | Low active. Synchronous sync on rising edge of CLK_YL 200 ns setup time. |
| 28 | SYL | Activate YL shift register for driving of reset and select line of pixel array. | High active. Exact pulsing pattern see timing diagram. Both SYR = 1 and SYL = 1 is not allowed, except when the same row is selected! |
| 29 | CLK_YL | Clock of YL shift register. | Shifts on rising edge. |
| 30 | VDDR | Power supply for reset line drivers (= pin 21) | Nominal 4 V. Connected on-chip to pin 21. |
| 31 | VDD | Power supply. | Nominal 3.3 V |
| 32 | GND | Ground. | 0 V |
| 33 | GNDAB | Anti-blooming reference level (=pin 33). | Typ. 0 V. Set to 1V for improved anti-blooming. |
| 34 | SELECT | Control select line of pixel array. | High active. See timing diagrams. |
| 35 | RESET | Reset of the selected row of pixels. | High active. See timing diagrams. |
| 36 | CBIAS | Bias current column amplifiers. | Connect with 22 k to VDD and decouple with 100 nF to GND. |
| 37 | PCBIAS | Bias current. | Connect with 22 k to VDD and decouple with 100 nF to GND. |
| 38 | DIN | Serial data input. | 16-bit word. LSB first. |
| 39 | SCLK | SPI interface clock. | Shifts on rising edge. |
| 40 | CS | Chip select. | Data copied to registers on rising edge. |
| 41 | PC | Row initialization pulse. | See timing diagrams. |
| 42 | SYNC_X | Sets the X shift register to row 1. | Low active. Synchronous sync on rising edge of CLK_X 150 ns setup time. |
| 43 | GND | Ground. | 0 V |
| 44 | VDD | Power supply. | Nominal 3.3 V |
| 45 | CLK_X | Clock of YR shift register. | Shifts on rising edge. |
| 46 | SHR | Row track & hold reset level (1=hold; 0=track). | See timing diagram. |
| 47 | SHS | Row track & hold signal level (1=hold; 0=track). | See timing diagram. |
| 48 | XBIAS | Bias current X multiplexer. | Connect with 10 k to VDD and decouple with 100 nF to GND. |
| 49 | ABIAS | Bias current pixel array. | Connect with 10 M to VDD and decouple with 100 nF to GND. not used. may also be disconnected or grounded |

# Geometry & mechanical

## Die geometry

Figure 9: Die geometry and location of pixel (0,0)

## Pin number assignment

Pin 1 side identifier

Figure 10: pin number assignment. “Solid” drawn pins are connected to die attach area for a proper ground plane.

## Cover glass

Figure 10: S8612 transmittance curve (w/o AR coating)

Specification

- AR coating: 400-690 nm R < 1.5%

- Dig, haze, scratch 20 µm after coating

- Substrate: Schott S8612 glass

- Thickness: 0.7 mm +/- 0.050 mm

- Size: 31.9 x 44.9 mm2 +/- 0.2 mm

Defects (digs, scratches) detected at final test using F/11 light source. Glass defects that do not generate non-correctable pixels are accepted.

# Appendix 1: drawings

(package and cover glass drawings to be included in final pdf file)